# FireLink Receiver Node Documentation

## Overview

The receiver node listens for LoRa packets from the sender. Upon receiving a FIRE alert, it activates the LED and buzzer to warn users instantly.

## Purpose

This module is part of the FireLink LoRa-Based Real-Time Fire Detection and Early Warning System.

## Hardware Components

ESP32 DevKit V1

LoRa RA-02 (SX1278)

LED

Active Buzzer

Breadboard

Jumper Wires

## Pin Connections

| Component | ESP32 Pin |
| --- | --- |
| LED | GPIO26 |
| Buzzer | GPIO25 |
| LoRa NSS | GPIO5 |
| LoRa SCK | GPIO18 |
| LoRa MISO | GPIO19 |
| LoRa MOSI | GPIO23 |
| LoRa RST | GPIO14 |
| LoRa DIO0 | GPIO2 |

## Working Principle

Receiver continuously listens for LoRa packets.

FIRE message is received.

ESP32 activates LED and buzzer.

System resets when SAFE message is received.

## Features

LoRa long-range communication

Low power operation

Reliable real-time performance

Instant LED indication

Audible buzzer alert

## Code’s

#include <SPI.h>

#include <LoRa.h>

// ---------- LoRa ----------

#define LORA_SS    5

#define LORA_RST   14

#define LORA_DIO0  2

// ---------- Output ----------

#define LED_PIN     26

#define BUZZER_PIN  25

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

SPI.begin(18, 19, 23, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {

Serial.println("LoRa Failed!");

while (1);

}

Serial.println("Receiver Ready");

}

void loop() {

int packetSize = LoRa.parsePacket();

if (packetSize) {

String message = "";

while (LoRa.available()) {

message += (char)LoRa.read();

}

int rssi = LoRa.packetRssi();

Serial.print("Received : ");

Serial.print(message);

Serial.print(" | RSSI : ");

Serial.println(rssi);

if (message == "FIRE") {

digitalWrite(LED_PIN, HIGH);

tone(BUZZER_PIN, 1000);

}

else {

digitalWrite(LED_PIN, LOW);

noTone(BUZZER_PIN);

}

}

}

## Circuit Diagram