# FireLink Sender Node Documentation

## Overview

The sender node continuously monitors the flame sensor. When fire is detected, the ESP32 immediately transmits a FIRE alert through the SX1278 (RA-02) LoRa module to the remote receiver.

## Purpose

This module is part of the FireLink LoRa-Based Real-Time Fire Detection and Early Warning System.

## Hardware Components

ESP32 DevKit V1

LoRa RA-02 (SX1278)

Flame Sensor

Breadboard

Jumper Wires

USB Power

## Pin Connections

| Component | ESP32 Pin |
| --- | --- |
| Flame Sensor DO | GPIO27 |
| LoRa NSS | GPIO5 |
| LoRa SCK | GPIO18 |
| LoRa MISO | GPIO19 |
| LoRa MOSI | GPIO23 |
| LoRa RST | GPIO14 |
| LoRa DIO0 | GPIO2 |

## Working Principle

ESP32 reads the flame sensor.

Fire is detected.

LoRa transmits a FIRE message.

Returns to monitoring mode after transmission.

## Features

LoRa long-range communication

Low power operation

Reliable real-time performance

Real-time flame detection

Instant alert transmission

## Code’s

#include <SPI.h>

#include <LoRa.h>

// ---------- LoRa Pins ----------

#define LORA_SS    5

#define LORA_RST   14

#define LORA_DIO0  2

// ---------- Fire Sensor ----------

#define FIRE_PIN 27

void setup() {

Serial.begin(115200);

pinMode(FIRE_PIN, INPUT);

SPI.begin(18, 19, 23, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {

Serial.println("LoRa Failed!");

while (1);

}

Serial.println("Fire Sender Ready");

}

void loop() {

bool fire = digitalRead(FIRE_PIN);

if (fire == LOW) {      // LOW means fire detected on most modules

Serial.println("FIRE DETECTED");

LoRa.beginPacket();

LoRa.print("FIRE");

LoRa.endPacket();

delay(1000);

}

else {

Serial.println("SAFE");

LoRa.beginPacket();

LoRa.print("SAFE");

LoRa.endPacket();

delay(2000);

}

}

## Circuit Diagram